#include <WiFi.h>

#include <PubSubClient.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT22

#define SOIL_PIN 34

#define PUMP_PIN 2

const char* ssid = "Wokwi-GUEST";

const char* password = "";

const char* mqtt_server = "broker.hivemq.com";

const char* TOPIC_DATA = "wit37/irrigation/data";

const char* TOPIC_PUMP = "wit37/irrigation/pump";

const int SOIL_THRESHOLD = 40;   // % below this = dry

WiFiClient espClient;

PubSubClient client(espClient);

DHT dht(DHTPIN, DHTTYPE);

String mode = "AUTO";            // AUTO, ON, OFF

bool pumpOn = false;

unsigned long lastSend = 0;

void publishState() {

int raw = analogRead(SOIL_PIN);

int soil = map(raw, 0, 4095, 0, 100);   // 0 = dry, 100 = wet

float t = dht.readTemperature();

float h = dht.readHumidity();

if (isnan(t) || isnan(h)) { t = 0; h = 0; }

if (mode == "AUTO") pumpOn = (soil < SOIL_THRESHOLD);

else pumpOn = (mode == "ON");

digitalWrite(PUMP_PIN, pumpOn ? HIGH : LOW);

char buf[160];

snprintf(buf, sizeof(buf),

"{\"soil\":%d,\"temp\":%.1f,\"hum\":%.1f,\"pump\":%s,\"mode\":\"%s\"}",

soil, t, h, pumpOn ? "true" : "false", mode.c_str());

client.publish(TOPIC_DATA, buf);

Serial.println(buf);

}

void callback(char* topic, byte* payload, unsigned int length) {

String msg = "";

for (unsigned int i = 0; i < length; i++) msg += (char)payload[i];

msg.trim();

msg.toUpperCase();

if (msg == "ON" || msg == "OFF" || msg == "AUTO") {

mode = msg;

Serial.println("Command received: " + msg);

publishState();              // apply and report immediately

lastSend = millis();

}

}

void connectWiFi() {

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println(" connected");

}

void connectMQTT() {

while (!client.connected()) {

String id = "esp32-wit37-" + String(random(0xffff), HEX);

Serial.print("Connecting to MQTT...");

if (client.connect(id.c_str())) {

Serial.println(" connected");

client.subscribe(TOPIC_PUMP);

} else {

Serial.print(" failed, rc=");

Serial.println(client.state());

delay(2000);

}

}

}

void setup() {

Serial.begin(115200);

pinMode(PUMP_PIN, OUTPUT);

dht.begin();

connectWiFi();

client.setServer(mqtt_server, 1883);

client.setKeepAlive(60);

client.setCallback(callback);

}

void loop() {

if (!client.connected()) connectMQTT();

client.loop();

if (millis() - lastSend >= 5000) {

lastSend = millis();

publishState();

}

}